Technical Reference Guide

Transport Layer

Troubleshooting

A  reference for diagnosing and resolving common issues at the Transport Layer

Prepared by

Aiden Martin

Department / Team

Network Engineering

Date

April 2026

Version

1.0

TCP/IP UDP Port Management, Congestion Control, TLS/SSL Wireshark

Table of contents

- Common problem 1: Connection Establishment Failures (TCP)

- Common problem 2: Port Conflicts & Unavailability

- Common problem 3: TCP Retransmissions & Packet Loss

- Common problem 4: Flow Control Issues

- Less common problem 1: TCP Silly Window Syndrome

- Less common problem 2: Ephemeral Port Exhaustion

- Less common problem 3: TCP ACK Compression / ACK Thinning

- Less common problem 4: Receive Buffer Auto-Tuning Interference

- Connection Establishment Failures (TCP)

Problem: The sender overwhelms the receiver — TCP receive window shrinks to zero, stalling the connection.

Solutions:

- Increase socket receive buffer sizes (SO_RCVBUF)

- Ensure TCP window scaling is enabled (especially over high-latency links)

- Check for a slow application consuming data from the buffer

- Port Conflicts & Unavailability

Problem: A service can't bind to a port because it's already in use, or a client can't reach a port that's blocked.

Solutions:

- Use netstat, ss, or lsof -i to identify what's occupying a port

- Change the application's configured port

- Review firewall/ACL rules on routers and host-based firewalls

- TCP Retransmissions & Packet Loss

Problem: Segments are lost in transit, causing slow performance and retransmission storms.

Solutions:

- Use Wireshark or tcpdump to identify retransmission patterns

- Check physical/link layer for errors (bad cable, faulty NIC)

- Tune TCP retransmission timeout (RTO) values

- Reduce congestion on the network path

- Flow Control Issues

Problem: The sender overwhelms the receiver — TCP receive window shrinks to zero, stalling the connection. Solutions:

- Increase socket receive buffer sizes (SO_RCVBUF)

- Ensure TCP window scaling is enabled (especially over high-latency links)

- Check for a slow application consuming data from the buffer

### TCP Silly Window Syndrome (SWS)

Problem: The receiver advertises tiny window sizes (sometimes just a few bytes), causing the sender to transmit many small, inefficient segments instead filling a proper-sized packet. Solution:

- On the receiver side, apply the Clark's Algorithm — don't advertise a new window until you can offer at least half the full buffer size or one MSS (Maximum Segment Size), whichever is smaller.

- On the sender side, apply Nagle's Algorithm — buffer small outgoing writes and only send when either a full MSS is ready or all previous data has been acknowledged.

- Note: Nagle's Algorithm can conflict with latency-sensitive apps (like games or SSH). In those cases, disable it with TCP_NODELAY and handle SWS at the application level instead.

- Ephemeral Port Exhaustion

Problem : High-throughput systems  like load balancers or scrapers making thousands of outbound connections per second can burn through all available ports, causing new connections to fail silently with "cannot assign requested address".

Solution:

- Expand the ephemeral port range: sysctl -w net.ipv4.ip_local_port_range="1024 65535"

- Enable net.ipv4.tcp_tw_reuse=1 to recycle TIME_WAIT sockets faster

- Use connection pooling to reuse existing connections rather than opening new ones constantly

- Distribute outbound traffic across multiple source IPs if scaling further

### TCP ACK Compression / ACK Thinning

Problem: deliberately collapsing or delaying multiple ACKs into one to reduce upstream traffic. This distorts TCP's pacing and congestion signals  the sender misreads the ACK patterns, causing jitter and unpredictable throughput drops. Solution:

- Identify with Wireshark — look for ACKs arriving in irregular bursts rather than a steady stream

- Enable TCP RACK (Recent ACK) on Linux (net.ipv4.tcp_recovery=1), which uses timestamps rather than ACK counting for loss detection, making it more resilient to ACK manipulation

- Use QUIC/HTTP3 for affected traffic — since QUIC handles its own acknowledgment logic over UDP, it bypasses middlebox interference entirely

- If the middlebox is under your control, disable ACK thinning in its WAN optimization settings

### Receive Buffer Auto-Tuning Interference

Problem: Modern OSes dynamically resize TCP receive buffers to match throughput (Linux's tcp_moderate_rcvbuf). However, on certain high-latency or asymmetric links, the auto-tuner reacts too slowly or overshoots causing the receive window to fluctuate wildly. The result is a connection that performs well briefly, then stalls, then recovers in cycles, with no obvious packet loss to explain it. Solution:

- Diagnose by graphing the TCP window size over time in Wireshark erratic sawtooth patterns with no corresponding loss events point to buffer mistuning

- Temporarily disable auto-tuning and set manual buffer sizes to confirm: sysctl -w net.ipv4.tcp_moderate_rcvbuf=0

- If confirmed, tune the min/default/max buffer values manually: net.ipv4.tcp_rmem = 4096 87380 16777216

- For very high BDP (bandwidth-delay product) links like satellite or intercontinental connections, significantly increase the max: net.ipv4.tcp_rmem = 4096 131072 67108864